CAPSTONE PROJECT REGISTER

Class:     Duration time:  from July 2026. To December, 2026

(*) Profession: <Software Engineer>   Specialty:      <ES>         <IS>         <JS>

(*) Kinds of person make registers:         Lecturer 		Students

## 1. Register information for supervisor (if have)

|  | Full name | Phone | E-Mail | Title |
| --- | --- | --- | --- | --- |
| Supervisor |  |  |  |  |

## 2. Register information for students (if have)

|  | Full name | Student code | Phone | E-mail | Role in Group |
| --- | --- | --- | --- | --- | --- |
| 1 | Nguyễn Chí Bảo | SE183383 | 0395239426 | baoncse183383@fpt.edu.vn | Leader |
|  | Trần Minh Kiệt | SE182180 | 0912213070 | kiettmse182180@fpt.edu.vn | Member |
| 2 | Nguyễn Vũ Len | SE182055 | 0974600126 | lennvse182055@fpt.edu.vn | Member |
| 4 | Nguyễn Thị Yến Nhi | SE184223 | 0938283598 | nhintyse184223@fpt.edu.vn | Member |
|  | Thái Minh Tuấn | SE181850 | 0942450156 | tuantmse181850@fpt.edu.vn | Member |

## 3. Register content of Capstone Project

## 3.1. Capstone Project name:

- English: SwiftletCare: An Automated Environmental Control and Multi-Modal Health Monitoring System for Swiftlet Farming.

- Vietnamese: SwiftletCare: Hệ thống tự động hóa điều khiển môi trường và giám sát sức khỏe nhà yến đa phương thức.

- Abbreviation: SwiftletCare

### A. Context:

### Commercial swiftlet farming is highly lucrative but extremely sensitive to environmental micro-climates (temperature, humidity, airflow, light intensity) and internal acoustics. Currently, most swiftlet farmers rely on manual inspection or disjointed, low-cost sensor devices that require frequent physical intervention. Maintaining improper humidity (<75%) or temperature (>31°C) leads to nest degradation, bird desertion, or high chick mortality rates.

### Moreover, continuously tracking swiftlet population dynamics (the daily difference between birds exiting and returning to the house), as well as early detection of disease outbreaks, predator intrusion (rats, snakes, owls), or sound playback equipment failures, is virtually impossible without continuous automated monitoring. Therefore, an integrated, low-cost IoT ecosystem combined with edge processing (Edge AI) and cloud analytics is urgently needed to automate environmental regulation, monitor swiftlet population fluctuations via computer vision, lower operational overhead, and safeguard swiftlet yields.

### B. Proposed Solutions:

### We propose SwiftletCare, a comprehensive IoT system combining hardware edge units, automated actuators, an AI-powered camera vision module, and a centralized web/mobile platform.

### The core research focus and value proposition lies in the Multi-Modal Automated Micro-Climate and AI Anomaly Detection Engine:

### Micro-Climate Control:

### Instead of relying solely on reactive manual controls, the system continuously reads multi-point sensor data (DHT22/SHT31, CO2, Light) and uses an adaptive closed-loop control algorithm (PID/Rule-based) to automatically trigger misting systems, ventilation fans, and heaters.

### Swiftlet Entry/Exit AI Counting (Edge/Cloud Computer Vision):

### Tapping lightweight visual detection and tracking models (YOLO / Lightweight CNN + Object Tracking) at the swiftlet entry/exit roaming holes to automatically count and track bird outflow (morning) and inflow (evening), providing data-driven insights into flock growth or desertion.

### Multi-Modal Threat Detection:

### Edge nodes combine acoustic signal processing (bird call patterns vs. distress/predator sounds) and visual cues (detecting predators like rats, snakes, or owls) to instantly alert farm owners in real time while minimizing cloud processing bandwidth and server costs.

### C. Functional requirement:

### 1. Farm Owner / Administrator:

### Device & Zone Management: Register, configure, and monitor multiple swiftlet houses, AI camera nodes, or distinct floors/zones.

### Automated & Manual Override Control: Set environmental thresholds (Temperature, Humidity, Light) for automatic actuator control, with a manual override mode via Web/Mobile UI.

### Swiftlet Entry/Exit Tracking & Live View: View real-time statistics on swiftlet exit/entry counts, stream live video feeds from camera nodes, and inspect captured anomaly frames (with bounding boxes).

### Real-Time Monitoring & Alerts: View live telemetry dashboards and receive instant push/SMS/Zalo notifications regarding critical threshold breaches, hardware failures, predator intrusion, or sudden flock size drops.

### Analytics & Yield Tracking: View historical environmental trends, nest growth logging, daily flock movement analytics, and correlation reports between environmental stability and harvest productivity.

### 2. System (Background Services & Edge Processing):

### Sensor Telemetry Ingestion: Collect, validate, and process high-frequency IoT data transmitted via MQTT/HTTP protocols.

### Automated Actuation Loop: Execute threshold and PID control logic to drive relays (misting devices, exhaust fans, heaters, acoustic amplifiers).

### Audio & Computer Vision Signal Processing:

### Process sound samples to ensure nesting/attractant sounds are playing properly and detect anomalous noise levels (e.g., equipment malfunction or distress calls).

### Camera AI & Bird Counting Engine: Run object detection and multi-object tracking (MOT) algorithms to count swiftlets entering and exiting the house entrance while detecting predator intrusions (rats, snakes, owls).

### D. Main flow:

### Flow 1: Edge Provisioning & Network Onboarding: The farm owner installs the IoT controller node and AI Camera, connects them to the local Wi-Fi/4G gateway, and pairs them with the SwiftletCare platform using QR onboarding.

### Flow 2: Real-time Telemetry & Closed-Loop Actuation: Edge sensors measure ambient temperature, humidity, and air quality every N seconds. If humidity drops below the target threshold, the edge controller immediately triggers the misting relay without needing server round-trip latency, while syncing status to the cloud.

### Flow 3: Audio & Equipment Health Monitoring: The system monitors the output of the sound playback units. If audio frequency/amplitude drops (speaker failure) or distress calls are detected, an urgent alert is logged and pushed to the user.

### Flow 4: Camera AI Detection & Swiftlet Counting: The camera mounted at the entrance/exit hole records bird traffic. The AI model detects and counts outgoing swiftlets (morning departure) and incoming swiftlets (evening return), computes the daily return rate, and syncs aggregated data to the cloud. If a predator (snake, rat, owl) is detected, the system immediately captures the frame and fires a high-priority alert.

### Flow 5: Dashboarding & Multi-Zone Analytics: Aggregated environmental metrics, swiftlet count charts, and camera feeds are visualized on the web/mobile dashboard, allowing owners to compare conditions across different floors and optimize nest yield.

### Flow 6: Incident Handling & Notification: When anomalies occur (e.g., predator detected via camera, sudden drop in returning swiftlets, temperature spike, water pump running dry, power outage), the system triggers priority alerts (App Push / Zalo ZNS / SMS) with actionable steps.

### E. Non-functional requirement:

### Reliability & Offline Resiliency: Edge nodes continue executing local automated control rules (misting/ventilation) even if the internet connection to the cloud server is temporarily lost.

### Performance: Real-time sensor data updates on the dashboard within 2 seconds using WebSockets/MQTT; swiftlet count metrics and camera anomaly alerts are delivered to mobile devices within 3–5 seconds.

### Cost-Efficiency & Bandwidth Optimization: Designed using affordable, robust hardware components (ESP32, standard industrial sensors, IP/ESP32-CAM nodes); optimized AI processing pipeline (transmitting aggregated count data and snapshot alerts rather than continuous video streams) to minimize 4G bandwidth consumption and cloud storage costs.

### 3.2. Main proposal content (including result and product)

a) Theory and practice (document):

  - Application of IoT Architecture patterns, Computer Vision & Deep Learning (Object Detection & Multi-Object Tracking), Sensor Fusion, Embedded Systems Programming, MQTT Protocol, and Closed-loop Control Systems.

  - Embedded/Hardware: ESP32, ESP32-CAM / IP Camera, FreeRTOS/Arduino framework, MQTT Protocol.

  - AI / Computer Vision: OpenCV, YOLOv8 (nano/lightweight), ByteTrack / DeepSORT (Object tracking).

  - Backend: NodeJS / NestJS, MongoDB, Redis, MQTT Broker (EMQX/Mosquitto), S3/MinIO (image snapshot storage).

  - Frontend/Mobile: React.js / Next.js, TailwindCSS, React Native.

b) Products:

  - An operational IoT Hardware Controller Prototype & AI Camera Node (ESP32-based    controller connected to sensors and relay modules + entrance vision module).

  - A trained AI Model for Swiftlet Entry/Exit Counting & Predator Detection.

  - A fully functional Web & Mobile Application for monitoring environmental data, viewing swiftlet count analytics, streaming camera feeds, and managing controls.

  - A scalable Cloud Backend handling real-time time-series telemetry, swiftlet counting logs, and alert dispatching.

c) Proposed Tasks for students:

Task 1: Design circuit schematics, assemble prototype boards, integrate camera modules, and code ESP32 firmware for sensor reading, relay control, and MQTT communication.

## Task 2: Collect/annotate datasets for swiftlets and predators, train lightweight AI models (YOLO + Object Tracking) for entry/exit counting, and integrate the vision pipeline.

## Task 3: Build Backend API (NodeJS/NestJS), set up MQTT Broker, design Time-Series Database schemas (MongoDB/PostgreSQL) for sensor telemetry & count logs, and integrate WebSocket channels.

## Task 4: Build Web Dashboard (React.js/Next.js) for administrative oversight and Mobile App (React Native/Flutter) for swiftlet counting analytics and real-time alert notifications.

## Task 5: Deploy empirical system runs in a simulated/real swiftlet environment, evaluate the accuracy of the swiftlet counting model (Precision/Recall), analyze response latency, test, and write the thesis/scientific paper.

## 4. Other comment (propose all relative things if have)

The hardware prototype will be built with fail-safe hardware watchdogs and physical bypass switches so that farm owners can manually operate fans/misting pumps in the event of total hardware or power failure.

Entrance AI Camera modules will support High FPS capture and Infrared Night Vision to reliably detect and count fast-moving swiftlets during early morning departures and late evening returns under low-light conditions.

| Supervisor (If have) (Sign and full name) | HCM, date 22/07/2026 On behalf of Registers (Sign and full name) |
| --- | --- |